Cow-Boy Game

Jaba Chkhikvadze

Microprocessors peripheral units:

- Writing the RISC-V assembler code for the device,that was specified by the instructor.

laboratory assignments #10

### 1. Objective

The purpose of this lab was to implement a simple reaction-based game using a custom-built RISC-V processor simulated in C. The processor interacts with peripherals by executing store, load, and comparison instructions. The experiment focused on writing C functions to encode and simulate the execution of these RISC-V instructions within a host-controlled environment, integrating both user input and memory-mapped peripheral logic.

### 2. System Overview

The system simulates a two-player "reaction game" where each player responds to a stimulus by pressing a key. Their reaction times are recorded and passed to the RISC-V processor, which stores the values in memory and determines the winner by comparing the two times.

The architecture comprises two primary components:

- C Code: Responsible for handling real-time input/output operations, capturing timestamps using QueryPerformanceCounter, and invoking the RISC-V simulation.

- Simulated RISC-V Processor: Executes encoded instructions to manipulate data, store values in memory, and compare results.

### 3. Memory Map

The game utilizes specific memory locations to represent peripheral registers. These are simulated via a word-addressable array (Data_Memory[]), where each index corresponds to address >> 2:

| Memory Address | Register Purpose | Data_Memory Index |
| --- | --- | --- |
| 0x2000 | Player 1 Reaction Time | 0x800 |
| 0x2004 | Player 2 Reaction Time | 0x801 |
| 0x2008 | Comparison Result (slt) | 0x802 |
| 0x200C | Game Status (Unused/Optional) | 0x803 |

### 4. Control Logic and Instruction Decoding

The simulated processor's control unit supports the decoding and execution of several key RISC-V instruction formats, specifically:

- R-Type: Used for operations like slt (set less than)

- I-Type: Used for instructions like addi (add immediate)

- S-Type: Used for sw (store word)

- Load/Store: lw and sw for memory interactions

- Branch and Jump: beq, jal for conditional logic (not utilized in this lab)

Instruction decoding is handled by Main_decode() and ALU_decode() functions. These generate control signals (e.g., RegWrite, ALUSrc, ResultSrc, MemWrite, etc.) that dictate the datapath behavior.

### 5. Reaction Time Storage Function

The function risc_v_store_player_time(int player, int reaction_time) encodes and simulates the following sequence:

- Load reaction time into register x5:

  - Instruction: addi x5, x0, [reaction_time]

- Load peripheral address into x6:

  - Instruction: addi x6, x0, [address]

  - Address determined by player number

- Store x5 into memory at address held in x6:

  - Instruction: sw x5, 0(x6)

These instructions are encoded manually using bitwise operations to create valid 32-bit RISC-V opcodes and then passed to execute_instruction() for simulation.

### 6. Winner Determination Logic

The comparison function risc_v_compare_and_determine_winner() executes the following steps:

- Load player reaction times into x7 and x8:

  - Instructions:
lw x7, 0x2000(x0) (Player 1)
lw x8, 0x2004(x0) (Player 2)

- Compare using slt (set if less than):

  - Instruction: slt x9, x7, x8

  - Sets x9 to 1 if Player 1 was faster, else 0

- Store result to comparison memory:

  - Instruction: sw x9, 0x2008(x0)

The C host then reads the result from memory to determine and display the game outcome. If the values are equal, a draw is declared.

### 7. RISC-V Instruction Encoding

Each instruction is encoded by manually setting the correct fields: opcode, destination register (rd), source registers (rs1, rs2), funct3, and funct7.

Example – Store Instruction (sw x5, 0(x6)):

c

uint32_t sw_instruction =

((0 & 0xFE0) << 20) |  // imm[11:5]

(5 << 20) |            // rs2 = x5

(6 << 15) |            // rs1 = x6

(0b010 << 12) |        // funct3 = 010

(0 << 7) |             // imm[4:0]

0b0100011;             // opcode = sw

Example – Set Less Than (slt x9, x7, x8):

c

uint32_t slt_instruction =

(0 << 25) |    // funct7

(8 << 20) |    // rs2 = x8

(7 << 15) |    // rs1 = x7

(0b010 << 12) |// funct3 = 010

(9 << 7) |     // rd = x9

0b0110011;     // opcode = R-type

### 8. C–Processor Interaction Flow

Below is the typical interaction flow between the host system and the processor:

- User presses a key

- Reaction time is calculated via performance counters

- risc_v_store_player_time() is invoked to store the time into memory

- When both players have acted, risc_v_compare_and_determine_winner() is called

- The result is read back and displayed by the host

This structure clearly separates low-level instruction execution (RISC-V) from higher-level game logic and I/O (C environment).